REQUEST FOR PROPOSAL
SPECIFICATION OF SPECIFIED GOODS

Procurement No:	46-G009-21

## Specification

### Background

This project is to purchase a suitable Landing Craft that is in operational condition, which after purchase, will be able to commence transporting petroleum products in drums and tanks to the outlying islands in Kiribati. It is critical that the technical specification for the Landing Craft, listed below, is followed.

The main focus is to address the longstanding issue of fuel shortage in the islands, hampering development, due to lack of shipping.

The landing craft will also be able to uplift copra from the islands to Tarawa and similarly address the longstanding issue of uncollected copra due to lack of shipping.

### Requirements

All supporting documentation must be in English.

- General Arrangement (GA)

- Technical Specification

- List of mandatory certificates and dates of expiry

- Other related documents

### Installation services

If not already fitted, the following items will need to be installed:

- 3 to 4 tons deck crane to uplift fuel Drums and workboats.

- 3 x 7 metre Heavy duty aluminium work boats (detail below)

- 3 x 40 hp 4 stroke outboard motors

- Water maker – about 2 tons per day

### Delivery Time

Please specify delivery time

Once agreement is signed and required equipment are fitted.

## Description of the Goods

Here, list all items to be Tendered

(This part may be replaced by a proprietary Supplier description)

Listed below is the technical specification for a Landing Craft to suit the requirements of KOIL.

| PART 1: General Specifications |  |
| --- | --- |
| Ship’s Principal Particulars | Description |
| Vessel Type | Landing Craft |
| Launched | No later than 2014 |
| Length | Less than 50 metres |
| Beam | Up to 12m |
| Designed draft | Between 1.8m and 2.6 metres (Maximum) |
| Gross Tonnage | Less than 500 |
| Deadweight | Up to 500 tons on draft 2.5m |
| Deck Strength | 5 Mt/Sqm |
| Containers | Up to 24 with 6 or more reefer points |
| Double Bottom Fuel Tanks | Will be advantageous but not necessary |
| Operational status | Currently operating |
| Classification - 100a1 LMC | Lloyds Register or approved IAS member |
| Moulded Depth | Maximum 3.20m |
| Ramp | Approx 6m x 5.5m |
| Compliment | Minimum 12 Crew |
| Main engines (Notable brand) | 2 x approx 829ps each |
| Generators (Notable brand) | 3 x approx 78kw |
| Bow Thruster | 3T – Engine or motor driven |
| Speed (approx) | 10-11knots on 4000litres MDO/day |
| Ship’s Fuel (approx) | 100,000 litres |
| Cargo Fuel (approx) | 300,000 litres |
| Fresh Water (approx) | 145,000 litres |
| PART 2: Additional requirements |  |
| - Flat Bottom - No appendage below the keel line including NO skeg keel (GA must be included) |  |
| - Steering System must incorporate main and auxiliary steering gear |  |
| - Scantling (Minimum) - Keel: 12 mm - Bottom: 10 mm - Hull: 10 mm - Deck: 8 mm - Partition: 8 mm - Frame: 8 mm - Strip: 8 mm x 7.5 cm – 6 mars |  |
| - Air Conditioning: Split system and cooling providing redundancy throughout the vessel |  |
| - Safety Equipment - Standard for coastal & offshore operation |  |
| - Navigation Equipment - Standard for coastal & offshore operation |  |
| PART 3 : THE FOLLOWING ITEMS ARE TO BE FITTED ON AGREEMENT TO SELL, IF NOT ALREADY FITTED |  |
| - Deck Crane – about 4 tons |  |
| - 3 x 7m aluminium heavy-duty workboats with built-in double bottom, self-draining deck and Engine well, a bow ramp 1.5 metres wide with lifting lugs. - 3 x 40hp 4 stroke outboard Motors to power the workboats. |  |
| - Water maker – about 2 tons per day |  |
| - Standard tools and operational spares for 3 months |  |
| - Passenger capacity desirable but not mandatory |  |

## Tenderer’s References

### Relevant similar deliveries carried out in the last five years

Please, provide information on each delivery for which your firm/entity, either individually as a corporate entity or as one of the major companies within an association, was legally contracted.

| Goods delivered | Buyer | Contact details | Value |
| --- | --- | --- | --- |