REQUEST FOR QUOTATION
SPECIFICATION OF STANDARD GOODS

Procurement No: 15-G002-22

## Specification

### Background

Water and Sanitation project is a 2009 first priority. Initially, the Permanent living quarter was considered the first priority for 2009. However, since the KPC Convention will be held on Arorae Island in August 19, 2014, the Council has adjusted its priority due the needs of toilet blocks to participants.  Later, the Island council sees the necessity of utilizing the remaining fund for water and sanitation to build a three Permanent living quarters for Civil Servants at the Council Station.

Currently, the Island Council has six existing old local buildings that require maintenance most of the time. The number of existing Living Quarters could not cater as well for all civil servants and this leads them to live outside the Council’s compound.

It is believed that by providing required permanent living quarters will allow civil staff to live in a more comfortable and safe home and to reduce future repeated maintenance of local buildings.  In addition, it will generate more revenue through renting and a great performance to the Council.

### Requirements

All supporting documentation must be in English. All suppliers are required to submit the following documents.

### Installation services

<insert related services>

### Delivery Time

Suppliers are required to provide the delivery timeframe.

## Description of the Goods

Here, list all items to be Tendered

(This part may be replaced by a proprietary Supplier description)

| Pos. | Description | Number | Quantity | Price (to be Tendered) |
| --- | --- | --- | --- | --- |
|  | Description | To be purchased |  |  |
|  | Preambles |  |  |  |
|  | Timbers & PVC pipes |  |  |  |
|  | Unless otherwise stated all timbers and pvc pipes to be purchased in 6m long size given for timbers are actual sizes, and need to be dressed as per specification given on drawings. |  |  |  |
|  | Reinforcement |  |  |  |
|  | All bar reinforcements are to be purchased in 6m long. The quantities given are inclusive for laps, as per spec. |  |  |  |
|  | Conrete works (cement type GP or GB) and to be 20MPA Concrete strength |  |  |  |
| 1 | Polythene damp proof membrane (60m x 2m wide) 0.2mm thick | 1 |  |  |
| 2 | Tie wire | 1 |  |  |
| 3 | Denso tape or similar for joining polythene | 1 |  |  |
|  | Temporary work |  |  |  |
|  | Form work |  |  |  |
| 5 | Cruide oil or formwork reducing agent | 1 |  |  |
|  | Bar Reinforcement (AS/NZS4671) |  |  |  |
| 6 | D10mm dia reinforcing steel (6m length) | 22 |  |  |
| 7 | R6mm dia reinforcing steel (6m length) | 14 |  |  |
| 8 | Reinforcing mesh SL 102 size:6mx2.4m wide (200x200x10) | 2 |  |  |
| 9 | Allow for bar chairs, or precast concrete biscuits as chairs for bars | 1 |  |  |
|  | Finishing Trim |  |  |  |
| 11 | 50x15 cornice | 6 |  |  |
| 12 | 35x25 moulded scotia | 5 |  |  |
|  | Roofing Materials |  |  |  |
| 13 | 0.55mm BMT colorbond ultra (in corrugated/trimdex) (12feet) 3430m | 20 |  |  |
| 14 | 0.55mm BMT Colourbond ridge cap 8ft | 6 |  |  |
| 15 | 0.55mm BMT colourbond eaves guttering with bracket as necessary | 12 |  |  |
| 16 | Stop ends | 2 |  |  |
| 17 | Outlet to gutter | 2 |  |  |
| 18 | 100MM diameter upvc downpipe including clips (in 6m long) | 2 |  |  |
| 19 | Clip for 100mm dia downpipe | 5 |  |  |
| 20 | 100mm diameter bend | 6 |  |  |
| 21 | 100mm diameter tee | 1 |  |  |
| 22 | Silicone | 2 |  |  |
| 23 | Rivert (100no/pkt) | 1 |  |  |
| 24 | Rivet gun | 1 |  |  |
| 25 | Hydroseal 400ml or similar | 1 |  |  |
|  | Doors and Window |  |  |  |
| 26 | Louver frame 6 blade | 24 |  |  |
| 27 | Louver frame 10 blade | 8 |  |  |
| 28 | 2100x900x40mm internal door single | 1 |  |  |
| 29 | 2100x900x40mm External solid core door single | 1 |  |  |
| 30 | 100x75mm butt brass hinges with screws | 6 |  |  |
| 31 | Plain mortice lock complete with handles and keys | 2 |  |  |
| 32 | 100 barrels bolt | 2 |  |  |
|  | Glazing |  |  |  |
| 33 | 24"x6" (610x150mm) clear glass blade | 224 |  |  |
|  | Lining |  |  |  |
| 34 | Hardboard lining 8"x4"x6mm thick for celing and wall | 23 |  |  |
| 35 | 9mm plywood sheet 8"x 4" | 4 |  |  |
| 36 | Chicken wire 15x2mtrs long | 2 |  |  |
| 37 | Sisalation alum. Foil one side fix over purlin and lapped at 300mm wide 15x 1.2mm long | 3 |  |  |
|  | Tiling |  |  |  |
| 38 | 300x 300mm vinyle tile 45pcs per box | 7 |  |  |
| 39 | Adhesive 4 liters | 12 |  |  |
|  | Painting and Decorating |  |  |  |
| 40 | 4" inch paint brush | 3 |  |  |
| 41 | 3 inch paint brush | 3 |  |  |
| 42 | 2 inch paint brush | 3 |  |  |
| 43 | wood putty | 5 |  |  |
|  | Solar light |  |  |  |
| 44 | 12v lead light 13 watt. (DC) | 3 |  |  |
| 45 | 12V SOLAR PANEL (190W) | 1 |  |  |
| 46 | Solar charge controller 12/24v 20/30A MPPT | 1 |  |  |
| 47 | Battery lugs/ Terminal | 2 |  |  |
| 48 | Light switch | 2 |  |  |
| 49 | 100 Ah Battery | 1 |  |  |
| 50 | DC cable | 30 |  |  |
| 51 | MC4 female with pins | 1 |  |  |
| 52 | MC4 male with pins | 1 |  |  |
| 53 | Cable clip | 1 |  |  |
|  | Nails, screws, bolt, etc (to be fully galvanised) |  |  |  |
| 54 | 3" galva nails | 6 |  |  |
| 55 | 2" galva nails | 8 |  |  |
| 56 | Hexagon roofing screws with EPDM washers | 4 |  |  |
| 57 | 4" Counter sunk wood screws @ 50/pkt | 24 |  |  |
|  | 5000ltrs water tank with concrete base |  |  |  |
|  | Preambles |  |  |  |
|  | Timbers and PVC PIPES |  |  |  |
|  | Unless otherwise stated all timbers and pvc pipes to be purchased in 6m long size given for timbers are actual sizes, and need to be dressed as per specification given on drawings. |  |  |  |
|  | Reinforcement |  |  |  |
|  | All bar reinforcements are to be purchased in 6m long. The quantities given are inclusive for laps, as per spec. |  |  |  |
|  | Concrete works (cement type GP or GB) and to be 20MPA |  |  |  |
|  | Bar reinforcement (AS/NZS4671) |  |  |  |
| 60 | D12mm dia reinforcing steel (6m length) | 4 |  |  |
| 61 | R6mm dia reinforcing steel (6m length) | 3 |  |  |
| 62 | R10 mm dia reinforcing steel (6m length) | 4 |  |  |
| 63 | Tie wire | 20 |  |  |
| 65 | 5000ltrs water tank | 1 |  |  |
| 66 | 20mm female adaptor pvc | 1 |  |  |
| 67 | 20mm pvc pipe | 1 |  |  |
| 68 | 20mm brass tap | 1 |  |  |
|  | Nails, Screws, Bolts, etc (to be fully galvanised) |  |  |  |
| 69 | 3" galva nails | 1 |  |  |
| 70 | 3" galva nails | 1 |  |  |