REQUEST FOR PROPOSAL
SPECIFICATION OF WORKS

Procurement No:	50-G004-22

Table of Contents

1.	Introduction	4

1.1	Background	4

1.2	About Kiribati Public Utilities Board	4

1.3	About The Ministry of Line and Phoenix Islands	4

1.4	Glossary of Terms	4

2.	About this RFP	5

3.	Scope of Supply	5

3.1	Prepay Meters	5

3.2	Prepay Management System	6

3.3	Vending Services	6

4.	Format of Proposals	7

5.	Stand-alone Prepay Meter Overview	7

5.1	Current State and Conceptual Future State Models	7

5.2	Future State Component summary	9

5.3	Core Components covered by this RFP	10

5.4	Standards	10

5.5	Service Functionality	10

5.6	Meters/Metering Equipment	11

5.6.1	Metrology	11

5.6.2	Configuration	11

5.6.3	Display	11

5.7	Interfaces	11

5.7.1	PUB IT Systems	11

5.8	Prepay Management System	12

5.9	Prepay Vending Services	12

5.9.1	Vending Channels	12

5.9.2	Vending System & Transactions	13

5.9.3	Offshore remittances	13

5.9.4	Cash Management and reconciliation	13

6.	Responsibilities	13

6.1	Reference Sites	13

6.2	Factory Acceptance Testing	13

6.3	Installation Support	14

6.4	Training	14

6.5	Documentation	14

6.6	Support Services	14

6.7	Spare Parts	15

6.8	Test Equipment	15

6.9	Supply Orders	15

6.10	Systems Commissioning	15

6.11	Warranties	16

6.12	Escrow	16

7.	Pricing	16

7.1	Equipment	16

7.2	Services	16

- Introduction

  - Background

Kiribati is an island country in the central Pacific Ocean. The permanent population is over 119,000 (2020), more than half of whom live on Tarawa atoll. The state comprises 32 atolls and one remote raised coral island, Banaba. There is a total land area of 811 square kilometres dispersed over 3.5 million km2 of ocean.  This RFP will apply to the Islands of South Tarawa and Kiritimati.

The Kiribati Public Utilities Board (PUB) mandate is for South Tarawa but it will be the procuring agent for the Ministry of Line and Phoenix Island Development (MLPID) in relation to electricity meters for Kiritimati.

  - About Kiribati Public Utilities Board

PUB is a corporate body, established under the Public Utilities Ordinance on 1 July 1977, and further amended in 2000 to allow PUB more autonomy in its operations. PUB was more recently established as an SOE under the SOE Act which came into force on 1 August 2013 with a more independent Board of Directors. The principal responsibilities of PUB are to deliver the following services to the population on South Tarawa of electricity, water and wastewater.

  - About The Ministry of Line and Phoenix Islands

The Ministry of Line and Phoenix Islands was established under the Government of Kiribati as a service provider for the region of Line and Phoenix groups. The Ministry has a significant role to enhance sustainable developments and leading to the high-end tourism development on Kiritimati Island, Tabuaeran, Teraina and Kanton Island.

  - Glossary of Terms

In this RFP, unless the context indicates otherwise:

Additional Services means additional services offered by a Proponent in their Proposal but that are not mandatory under the RFP;

Cash Management means the activity relating to the receiving of payments from PUB customers and the provision of the value of that payment to the PUB;

Contract means a supply contract that is entered into by the PUB and the Vendor, as a consequence of this RFP;

Customer means the end consumer of electricity;

Head-End means the meter head-end to be supplied by the Vendor to the PUB under the Contract having the functionality described in paragraph 6.8;

In Home Display means the in-home device used to key in token for stand-alone prepay meter solution;

Merchants mean the physical Point of Sale from which the PUB customers can procure;

Proponent means the person or persons who lodge a Proposal in accordance with the RFP;

PMS means a Prepay Management System;

Proposal means a Proponent's proposal submitted in response to, and in accordance with, this RFP;

PUB means Public Utilities Board (of Kiribati);

RFP means this request for proposals, including all appendices, any addenda and any Notices to Proponents;

Prepay Meters means the Prepay Meter that is used to deliver prepay services by having integration with Prepay Management System that normally uses STS token for secure top-up;

Support Services Agreement means the 15-year support services agreement to be entered into by PUB and the Vendor contemporaneously with the Contract;

Technical Requirements means the technical requirements set out in the Technical Requirements Overview in section 5 and the Technical Requirements in Appendix A;

Top Up means the purchase of a prepay electricity payment;

Vending channels means the channels through which the PUB customers could make a prepay electricity payment, such as a merchant, SMS, mobile wallet or internet banking;

Vending System means a central secure vending communications and database server that manages the receipt of vending transactions from vending channels, orchestration of transaction requests with financial clearing houses and the prepayment management system, controls vending finalisation and logs all transactions for reporting and audit; and

Vendor means the successful Proponent (if any) who enters into the Contract.

- About this RFP

This RFP has been developed in such a way as to enable PUB to establish a viable prepay electricity metering solution for use across Kiribati.  The RFP is based on a standalone prepay solution.

A high-level description of requirements and templates for Proponents’ responses is provided in Appendices A and B (as separate Word documents).

- Scope of Supply

This section sets out, at a high level, the scope of the system for which the PUB is seeking Proposals from Proponents.  The specific requirements are set out in the Functional Requirements (being section Appendix A). Proponents may respond to portions of the RFP rather than all of the AMI components described in the requirements section.

  - Prepay Meters

Through this RFP, the PUB is intending to enter into a long-term supply agreement with a stand-alone prepay metering vendor.

The prepay metering vendor can provide its own PMS or is able to integrate with a third party's PMS.

The number of meters purchased will depend on, amongst other factors, the price that the successful Proponent tenders.

The number of meters purchased could vary from 10,000 to 14,000 with possible ongoing annual quantities of approximately 500 meters.  The table below provides the current quantities of meters for South Tarawa and Kiritimati.

| South Tarawa Meters | Total |  | Kiritimati Meters | Total |
| --- | --- | --- | --- | --- |
| Single Phase, 1 Element | 11,953 |  | Single Phase, 1 Element | 1,520 |
| Three Phase, WC | 850 |  | Three Phase, WC | 73 |
| Three Phase, CT | 28 |  | Three Phase, CT | 0 |
| Totals | 12,831 |  | Totals | 1,593 |

Organic growth rates for the next three years for both locations are in the following table.

| South Tarawa Meters | Total |  | Kiritimati Meters | Total |
| --- | --- | --- | --- | --- |
| Single Phase, 1 Element | 2,253 |  | Single Phase, 1 Element | 421 |
| Three Phase, WC | 150 |  | Three Phase, WC | 36 |
| Three Phase, CT | 15 |  | Three Phase, CT | 0 |
| Totals | 2,418 |  | Totals | 457 |

  - Prepay Management System

The PUB has an existing PMS (DatelStream) that utilises STS tokens for stand-alone prepay meters. the PUB is interested in exploring additional options that would suit the Stand-alone Prepay Meter solution, or a PMS where the Smart Meter solution does not have prepay functionality embedded within the MDMS.

On this basis it will consider:

- Purchasing a license to existing PMS software

- Procuring the development of a fit for purpose bespoke PMS solution

- PMS software as a service (SaaS)

- Fully outsourced PMS service on a per meter or customer basis

Proponents should consider the scale of the PUB’s business and the appropriate fit for purpose PMS solution for a Pacific Island nation in their responses.

  - Vending Services

Through this RFI, the PUB is intending to identify appropriate service providers to work with for the supply of the vending components of its prepay electricity solution using AMI.

Specifically, the PUB is seeking proposals for:

Vending Channel Services

- The provision of a network of merchants able to provide electricity top up services to the PUB’s customers throughout Kiribati

- The provision of all the POS terminals and other technologies required to undertake prepay electricity top ups

- The provision of the messaging capability that interfaces to the PUB Prepay Systems for the financial transactions associated with each the PUB customer’s vending activity

- The provision of electronic vending channels appropriate for the PUB customers to top up pre-paid electricity accounts or meters

- The provision of an offshore remittance service for the direct top up of the PUB customers prepay electricity accounts

- The associated cash management, settlement, reconciliation, and reporting of payments made for prepay electricity by the PUB customers

- Format of Proposals

Proponents must submit all information required by this RFP in strict accordance with the forms and formats contained in Appendices A and B.  The reasons for this requirement are twofold:

- to make it easy for the PUB to evaluate and conform the responses; and

- to make it easy for Proponents to respond.

Appendices A and B are issued in editable Microsoft Word format documents along with this RFP.  Proponents should expand the response portions of tables to accommodate the information that they wish to provide to the PUB.  Information provided should allow a clear and detailed evaluation of the Proponent’s solution.

Proponents may elect to respond against certain portions of the requirements outlined in this RFP. Proponents are urged to complete the response forms fully of the components of the RFP they are responding against as the PUB may refuse to consider Proposals that do not provide all information required by this RFP or may score incomplete portions zero.  Any non-compliance with this RFP must be specifically highlighted by Proponents in their Proposals.  Non-complying Proposals may or may not be accepted by the PUB at its sole discretion.  Copies of brochures and other Vendor material are not required.

- Stand-alone Prepay Meter Overview

This section sets out, at a high level, the PUB requirements, including an indicative future state architecture and a description of the key underlying processes. Proponents should ensure their Proposal addresses and complies with both this section and the detailed Requirements for this RFP set out in Appendix A.  Appendix A also serves as a table of conformance.  Proponents should use the provided Microsoft Word format document of Appendix C to indicate their conformance with the Technical Requirements and all non-conformance and any departures/tags or commentary.

  - Current State and Conceptual Future State Models

The PUB’s current state at a high level is presented here so that the Proponent can identify areas of the PUB’s existing architecture that they could easily integrate with.

Figure 1 - the PUB High Level Current State

The PUB is only considering a Stand-alone Prepay Solution.  Smart meter solutions will not be considered.

Stand-alone Prepay Solution Conceptual Future State highlights the areas where Proponents are being asked to respond to, namely, the stand-alone prepay meter, the PMS and vending related services.

Figure 2 - Conceptual Future State Model for Stand-alone Prepay Solution

  - Future State Component summary

| Component | Owned by the PUB | Supply Arrangement Option | Scope for this RFP |
| --- | --- | --- | --- |
| Customer Premise Entity (CPE) – Stand-alone Prepay meters | YES | NO | YES |
| Vending Channels | TBC | Supplier Partnership | YES |
| Vending System | TBC | Supplier Partnership | YES |
| Prepayment Management System | YES | YES, can be via On-prem Licensed Software or Software as a Services (SaaS) or Outsourced back-office service provider and system arrangement | YES |
| Customer Communications Channel | YES | NO | YES |

  - Core Components covered by this RFP

The diagrams and table above show the components that are covered by this RFP. The components that are within the scope of this RFP are:

- Stand-alone Prepay Meter

- Prepayment Management System

- Vending System

- Vending Merchants/ Channels

  - Standards

Meter metrology must comply with the following International Standards

- IEC62052 Parts 11 and 21;

- IEC62053 Part 23; or

- ANSI C12.19.

- For Stand-alone Prepay - IEC 62044-41:2018 - Electricity metering - Payment systems - Part 41: Standard transfer specification (STS)-Application layer protocol for one-way token carrier systems (including version update to support STS2 by Nov 2024)

  - Service Functionality

Essential functional requirements are summarised as follows:

| Ref ID | Functional requirements | Stand-alone Prepay Meter Solution |
| --- | --- | --- |
| 1 | Remote capability to update tariff tables stored within the meter | Required via token |
| 2 | Remotely change tariffs | Required via token |
| 3 | Prepay (remote application of credit) | Required via token |
| 4 | Decrements credit amount per kWh consumption at the meter | Required |
| 5 | Auto disconnect/ reconnect at the meter | Required |
| 6 | Connection to in home display via physical wire or Zigbee | Required |
| 7 | Present warning of low credit or negative credit after emergency credit at meter and in-home display | Required |
| 8 | Present current load, remaining credit in dollars and estimated days at meter and in-home display | Required |
| 9 | Can update tariff structure or point to in-built tariff structure configured in the meter and likewise update other configuration such as non-disconnection window and emergency credit threshold (via key token) | Required |
| 10 | Can update rates via key token entry (to enable the PUB to increase or decrease rate) | Required |
| 11 | Capacity limiting | Required |
| 12 | Configurable levels of priority access to strategic devices (for network control and monitoring) | Required via port access |
| 13 | Ability to allocate meters/devices into multiple control groups (e.g. customer specific and regional) | Required |

  - Meters/Metering Equipment

    - Metrology

The detailed requirements for metrology are listed in Appendix A.

Proponents should note that an IEC form factor is required.

    - Configuration

Each order for meters will be accompanied with a configuration schedule indicating what configuration is to be programmed into the meters and which of the modular options are to be included into each type of meter.

    - Display

An in-home display unit will be required that enables customers to enter top-up tokens, receive alerts on low-credit or impending auto-disconnection and see current load usage and remaining credit in days and local currency.

  - Interfaces

    - PUB IT Systems

The PUB may wish to have specific interfacing developed to connect to their IT systems (e.g. billing, GIS, financial asset management) and SCADA systems (e.g. outage management, network switching, load control).  Proponents must indicate the feasibility of, and provide other appropriate information (including pricing) relating to, developing appropriate bespoke interfacing.  As a minimum, Proponents must indicate that they will provide sufficient interfacing details for the PUB or a third-party developer to develop an appropriate interface.

  - Prepay Management System

Figure 3 –Stand-alone Prepay Solution Payment Management System Core Functions

- Synch Master Data

  - Reliable and efficient master data synchronisation interface and process with the PUB’s Customer Information System including active tariff rates of prepay customers

  - Efficient and reliable importing of connection status information from the back-office system

- Vending Channel Authentication

  - Secure interface, identification, and authentication of partner vending channels for processing vending requests and providing responses

- STS Token generator

  - Generates STS compliant tokens (including ability to comply with STS2 by Nov 2023) either via vendor request/response or via UI by authorised users

- Reporting/ Exceptions Log

  - Reporting capability of transactions

  - Ability to review exceptions and error logs

- User Interface

  - UI for authorised user to generate report, generate token, update tariff mode

  - Prepay Vending Services

    - Vending Channels

- The provision of an extensive and accessible network of prepayment electricity merchants offering top up services for the PUB customers.

- Includes all associated telecommunications, terminals, printers, and other hardware necessary to provide vending services to meet the PUB’s requirements

- Any electronic vending channels able to be used remotely by the PUB customers such as SMS, Web vending, mobile wallet etc.

    - Vending System & Transactions

- The capability to request and receive vending token and type of vending requests to and from the prepay management system for stand-alone prepayment meter using STS certified interface protocol.

- The capability to securely generate financial transactions from Point of Sale (POS)  to the PUB’s pre-payment management system based on the purchases made by the PUB’s prepay electricity customers

- The ability to apply rules to ensure only approved/cleared financial transactions are processed with the Prepayment Management System

- Ability to present credit balance result and messages coming from the Prepayment Management System as part of finalising the Top-up

- The ability to report on transactions generated from all vending channels

    - Offshore remittances

- The capability for offshore payments to be made against the PUB customer’s prepay electricity accounts.

    - Cash Management and reconciliation

- The management of all cash, eftpos and credit card payments received from the PUB prepay electricity customers

- The settlement of payments in the PUB’s nominated bank accounts on a frequency that meets the PUB’s requirements

- The reconciliation and reporting of all payments against completed vending transactions

- Responsibilities

Once the Preferred Proponent(s) sign a Contract with the PUB for the respective components of this RFP, it (they) will become the Vendor(s).  This section sets out, at a high level, the responsibilities that the Vendor(s) will be required to undertake and, where applicable, the relevant information Proponents are required to provide.  More details of the Vendor(s)' responsibilities can be found in the Conditions of Contract in Appendix B.

  - Reference Sites

Proponents are required to provide a schedule of reference sites (Response Schedules, Appendix A) where the PUB can view their proposed solution working and discuss related matters with client organisations.  Preferred Proponents are required to give access to the reference sites listed in their Proposal to the PUB personnel as part of the evaluation process.

  - Factory Acceptance Testing

The PUB may wish to undertake or witness factory testing of the Vendor’s solution or part of that solution.  Proponents are to indicate the location for such testing in the Response Schedules (Appendix A).

  - Installation Support

The Vendor is required to be available at 48 hours' notice to provide on-site technical support during the initial installation phase of the meter roll-out programme which will be for a period of 3 months from delivery of the initial order and thereafter to provide non-urgent on-site support at 72 hours' notice.

Vendors that have special tools and software program(s) that will aid the installation process must provide details and pricing in the response Schedules, Appendix B.

All installation support costs are to be provided in the Response Schedules, Appendix B.

  - Training

The Meter Vendor is required to provide training to the PUB's (or the PUB Distributors') installation contractors on the correct methods of installation of all types of their meters one month ahead of the PUB’s start date for meter installation.  the PUB will give the Vendor one months' prior notice of the start date for meter installation.

The PUB requires that the Vendor(s) provides sufficient training to minimise any requirement on the PUB to use the Vendor’s staff on an ongoing basis.  Whilst some training may be required at the Vendor's factory it is expected that the majority of training will occur in Kiribati.

All training costs and venues are to be set out in the Response Schedules, Appendix B.  Ongoing support costs are to be priced separately from deployment and commissioning training.

  - Documentation

The Vendor(s) must provide all documentation required by the PUB to operate and maintain the systems provided by the Vendor (s).  This documentation shall be provided in electronic format and shall include, but shall not be limited to:

- Operating manuals;

- Maintenance manuals (both hardware and software and including any procedure for undertaking system upgrades);

- Installer manuals for meters;

- Commissioning test results;

- Meter and modem test certificates; and

- Third party product/software manuals (as applicable).

The Vendor(s) shall provide samples of the documentation which shall be approved by the PUB before acceptance.

  - Support Services

The Vendor(s) are required to provide support for their proposed solution for a period of not less than 15 years under a Support Services Agreement to be entered into between the PUB and the Vendor.

Proponents are required to detail the levels of support proposed and the associated annual costs in the Response Schedules, Appendix B.

First Level support will be provided by the PUB field service staff and Second Level will be provided by the PUB’s meter/communication specialists.

The PUB requires Third Level support to be provided by the Vendor.  This support may, on occasions, require site visits otherwise it is expected that most problems will be resolved using phone and email communications.

  - Spare Parts

Proponents are required to list any recommended spare parts required, other than meters, to be held by the PUB.  The Vendor is required to make spare parts available to purchase for a minimum period of 10 years.

Proponents are required to nominate quantities and prices for spare parts in the Response Schedules, Appendix B.

  - Test Equipment

Proponents should specify any test equipment essential for the installation and commissioning of the Head-End, meters and communications network that their commissioning personnel will not provide during the period of system commissioning.

Proponents should also specify any test equipment that is considered necessary for the PUB to own or to have access to in order to operate and/or maintain the meters, Head-End or communications network.

  - Supply Orders

The PUB intends to enter into a non-exclusive, long term (not less than 15 years) supply agreement with the Vendor to supply smart meters.  The Vendor will be required to supply interoperable meters for the duration of the Contract.

Most of the smart meters will be installed in residential premises but some agricultural, commercial and light industrial customers will also be supplied with 3 phase smart meters.

The approximate ratios of meter types are:

- Single phase – 93.5%

- Three phase whole current – 6.5%

The Vendor is only required to supply prepay meters upon receipt of an order to supply from the PUB.  Following the first supply order, the time from receipt of order to delivery in Kiribati shall be 3 months (subject to the volume of meters to be supplied in subsequent orders, which will be discussed with Short-Listed Proponents).

The PUB would like to have the first order supplied, in Kiribati during the 2nd quarter of 2023.  Proponents should indicate if this date is achievable and if not what the “firm” supply date would be.  Proponents should also indicate whether they can supply ahead of the required delivery date and provide the early delivery date.  Proponents should assume a minimum quantity of 2,000 meters for the initial shipment.  A small quantity, less than 20 meters, will also be required for testing and training purposes soon after contract execution.

Proponents should indicate their flexibility to respond to “just in time” requests for orders by reconfiguring meters in stock for shipment.

  - Systems Commissioning

The Vendor is required to take responsibility for commissioning the pre-payment and vending system.  During this period the Vendor’s commissioning personnel shall provide on-the-job training to the PUB personnel.

At the end of the commissioning process, the Vendor's lead commissioning engineer is required to certify that the system complies with the Technical Requirements and is fit for its purpose and ready for final acceptance testing.  Commissioning will be declared complete by the PUB project manager after reviewing the Vendor’s completion certificate and undertaking final acceptance testing.

  - Warranties

The Vendor(s) are required to warrant the reliable, safe and fit for purpose operation of all products supplied to the PUB.

The Vendor(s) are required to warrant that the services provided by the Vendor(s) will be rendered by qualified and competent personnel and that those services will be delivered in keeping with good industry practice for those services.

  - Escrow

Source code for any software that is specifically developed for the PUB by the Vendor(s) shall be delivered to the PUB (including source materials).  The source materials for any other software used in the Vendor's solution shall be placed in escrow.

- Pricing

  - Equipment

The Response Schedules, Appendix A requires Proponents to list their respective component prices.  The PUB will commit to a minimum order quantity (MOQ) of - meters over a minimum order period (MOP) of two years.  This commitment will be made in the Contract.  The PUB expects Proponents to factor this commitment into their pricing.

The unit meter prices to be paid for each new order for meters during the MOP will be fixed for the duration of the MOP.

There will be a price reset at the second and every subsequent anniversary of the Acceptance Date to consider any change in Vendor’s standard pricing but if the Vendor's standard list price is less than the adjusted price then the former will apply for the next 12 months.

  - Services

Services that relate directly to the procurement, installation and commissioning of meters are to be priced on a daily rate basis plus disbursements.  Post commissioning and system handover ongoing support services are to be priced as an annual fee or on a daily rate basis plus disbursements as is the Proponent's policy.

Vending services should be pricing on the basis of any upfront, annual fees together with transaction pricing.

Prices for Additional Services are to be priced on a fixed fee and/or daily rate basis plus disbursements.