REQUEST FOR QUOTATION
SPECIFICATION OF STANDARD GOODS

Procurement No: 46-G003-25

## Specification

### Background

This project is to enhance the efficiency of refuelling of aircraft at KOIL Aviation depot in Tab North.

### Requirements

All supporting documentation must be in English.

### Installation services

To be delivered in complete.

### Delivery Time

3 to 4 Months for Assemble of parts including & Delivery time to Tarawa Kiribati.

## Description of the Goods

Here, list all items to be Tendered

| Pos. | Description | Number | Delivery Time (to be Tendered) | Price (to be Tendered) |
| --- | --- | --- | --- | --- |
| 1 | 5000L Aviation Tanker Truck General Specifications - Tank Capacity: 5,000 liters (5 KL). - Brand: IZUSU or HOWO Truck Chassis: - Heavy-duty truck chassis suitable for aviation fuel operations. - GVW (Gross Vehicle Weight): Approximately 10-12 tons. - Minimum engine power: 180-220 HP. - Fuel type: Diesel. - Wheelbase: Optimized for maneuverability and stability. - Suspension: Heavy-duty, with shock absorbers suitable for off-road and on-tarmac use. - Tank Construction: - Material: Aluminum alloy or stainless steel (corrosion-resistant and compatible with aviation fuel). - Compartments: Single compartment with baffles to minimize fuel movement. - Certification: Compliant with international aviation fuel standards (e.g., ATA 103, JIG, EI/JIG 1530). - Insulation: Optional - Safety Features: - Emergency shut-off valves. - Overfill prevention system. - Static grounding system. - Fire extinguisher brackets and fire safety equipment. - Anti-skid platform and ladder with safety rails for operator access. Fuel Dispensing System 1. Underwing Refueling System - Flow rate: 50-300 liters per minute. - Refueling hose: 1-1.5 inches diameter, minimum 10 meters long, with aviation-grade couplings. - Nozzle: Pressure-controlled, aviation-grade nozzle (API or similar standard). - Filtration: Inline microfilter and water separator to ensure fuel cleanliness. 2. Overwing Refueling System - Flow rate: 50-150 liters per minute. - Refueling hose: 1-inch diameter, minimum 8 meters long. - Nozzle: Overwing pistol grip nozzle with a flow control trigger. - Swivel fittings: To prevent hose kinking during refueling. Pumping and Metering System - Pump: - Type: Aviation-grade centrifugal or gear pump. - Flow rate: 300-500 liters per minute (adjustable for underwing and overwing). - Power source: PTO (Power Take-Off) from the truck engine or auxiliary engine. - Metering System: - Type: Digital flow meter with calibration capability. - Accuracy: ±0.2%. - Readout: Clear digital display for volume, flow rate, and totalizer. - Hose Reels: - Spring-retractable or motorized reels for easy hose management. - Separate reels for underwing and overwing systems. Additional Features - Control Panel: - Location: Mounted for easy access by the operator. - Functions: Pump control, emergency shut-off, flow adjustments, and monitoring system. - Lighting: - LED floodlights for night operations. - Hazard lights for increased visibility during refueling. - Monitoring and Telemetry: - Optional system for real-time fuel inventory monitoring and data logging. - Toolboxes and Storage: - Secure compartments for tools, fittings, and documentation. - Markings and Labels: - Clear signage indicating fuel type, capacity, and hazard warnings. Regulatory Compliance - Adherence to ICAO, ATA, and local aviation authority standards. | 1 | 44 days after first payment |  |